# xFlow Document

Near Field Communication (NFC)

02/2026

Revision History

| Revision No. | Description | Editor | Date |
| --- | --- | --- | --- |
| 1.0 | Document Creation | Didar Ali | 09 Feb 2025 |

### Tables On Content

Near Field Communication (NFC)	3

Introduction	3

Definition Of NFC	3

Applications of NFC technology	3

Working Principle of NFC	3

NFC – Key Specifications	4

Communication Speed	4

Storage Capacity	4

Operating Range	5

Core Technologies and Tools in NFC	5

Standards	5

Hardware	5

Tools	5

Software	5

Encryption Keys in NFC	5

Challenges and Limitations of NFC	6

NFC Based ID cards and Passports	6

Introduction	7

Why NFC-Based ID Cards & Passports Are Used	7

Standards and Terminologies	7

Standards	7

Terminologies	8

How NFC-Based ID Cards and Passports Work	8

1. Data Storage in the Chip	8

2. Data Extraction	9

3. Authentication and Verification	9

4. Biometric Verification	9

5. Data Usage	10

6. Security Features	10

Reading NFC-Based e-MRTDs Using Mobile Phones	10

Steps for Reading	10

Data That Can Be Read	10

Limitations	10

Verification	11

Limitations of NFC-Based Passport Identification	11

NFC in Banking	11

Applications of NFC in Banking	12

1.	Contactless Card Payments	12

2.	Mobile Wallet Payments	12

3.	Cardless ATM Transactions	12

4.	Secure Customer Authentication	12

5.	Digital Onboarding and KYC Verification	13

6.	Virtual Card Issuance	13

7.	Access Control within Banking Infrastructure	13

8.	Transaction Authorization and Confirmation	13

Working Principle of NFC in Banking	13

Electromagnetic Field Generation	13

Inductive Coupling and Power Transfer	14

Data Exchange Mechanism	14

Encryption and Tokenization	14

Authentication	14

Transaction Flow Example – POS Payment	15

Transaction Flow Example – Cardless ATM	15

Security Features	15

Other Applications in Banking	15

Limitations of NFC	15

Conclusion	16

# Near Field Communication (NFC)

## Introduction

### Definition Of NFC

Near Field Communication is a hardware-dependent electromagnetic communication system that uses inductive coupling at 13.56 megahertz to support secure, short-range, bidirectional data exchange between active and passive devices.

### Applications of NFC technology

- Contactless payments: NFC enables fast and secure financial transactions using standardized payment protocols such as EMV. Users can complete payments by tapping NFC-enabled devices on point-of-sale terminals. Security is ensured through encryption, tokenization, and secure elements, preventing exposure of actual card details (e.g., Apple Pay and Google Pay).

- Data transfer: NFC supports short-range peer-to-peer communication for exchanging small data payloads such as contacts, URLs, credentials, or configuration data. Information is formatted using NFC Data Exchange Format (NDEF), enabling structured and interoperable data exchange with minimal setup and no pairing process.

- Secure identification and access control: NFC is widely used in authentication systems for secure access control, including office buildings, campuses, and events. Advanced NFC chips such as MIFARE DESFire implement strong cryptographic algorithms (DES, 3DES, AES) to provide tamper-resistant identity verification.

- Public transportation systems: NFC powers contactless transit cards and mobile tickets such as Oyster, Suica, and Metro cards. These systems use sector-based memory architecture and cryptographic security to manage fare deduction, balance updates, and transaction validation in real time.

- Marketing and information sharing: NFC tags embedded in posters, products, or packaging enable users to instantly access digital content such as websites, vCards, app links, or promotions. These tags use NDEF records, operate without batteries, and offer low-power, cost-effective user engagement solutions.

### Working Principle of NFC

- Electromagnetic field generation: An active NFC device (reader or terminal) generates an alternating magnetic field at 13.56 MHz through a coil antenna. This field exists in the near-field region and enables communication within a short range (typically up to 4 cm).

- Inductive coupling: When a second NFC-enabled device enters the magnetic field, inductive coupling occurs between the two antenna coils. A voltage is induced in the target device’s antenna, enabling signal detection and, in the case of passive devices, power harvesting.

- Power transfer to passive devices: Passive NFC devices, such as cards or tags, draw operating power from the magnetic field generated by the active device. This eliminates the need for an internal battery while maintaining reliable communication.

- Data transmission from reader to target: The active device transmits data by modulating the magnetic field (field modulation). The target device detects these variations and decodes them into digital commands.

- Data transmission from target to reader: The target device responds using load modulation, where it alters the load on its antenna circuit. These changes cause detectable variations in the reader’s magnetic field, allowing data to be sent back.

- Bidirectional communication: Through field modulation and load modulation, NFC supports two-way data exchange between devices, even when one device is fully passive.

- Protocol and data handling: Communication follows standardized NFC protocols, and exchanged information is processed using structured formats such as NDEF, ensuring interoperability and secure data interpretation.

- Mode-specific operation: NFC operates in multiple modes—reader/writer, card emulation, and peer-to-peer—each utilizing the same inductive coupling principle but differing in data flow and device roles.

## NFC – Key Specifications

### Communication Speed

NFC supports multiple data transfer rates:

- 106 kilobits per second (kbps)

- 212 kbps

- 424 kbps

The speed depends on the mode of operation and device capabilities; Faster rates (212 or 424 kbps) are typically used for peer-to-peer data exchange, while 106 kbps is often sufficient for tag reading/writing or payment transactions.

### Storage Capacity

NFC itself does not have a standard memory; storage depends on the type of NFC tag or card:

| NFC Tag Type | Storage Capacity | Notes |
| --- | --- | --- |
| Type 1 | 96–2,048 bytes | Simple read/write tags, low cost |
| Type 2 | 48–2,048 bytes | Widely used, e.g., smart posters |
| Type 3 | Up to 1 MB | High capacity, e.g., Sony FeliCa |
| Type 4 | Up to 32 KB | Often used for secure payments (EMV cards) |
| Type 5 | Up to 64 KB | ISO 15693 compatible, fast reading |

Smartphones can also store secure credentials in their Secure Element (SE) or embedded storage, which is independent of the NFC physical layer

### Operating Range

NFC is inherently short-range, designed for secure, close-proximity interaction:

- Typical range: 0–4 centimeters (≈1.5 inches)

- Maximum range in practical implementations: up to 10 centimeters, but reliability decreases with distance.

Short range is intentional to enhance security and reduce interference with other RF devices.

## Core Technologies and Tools in NFC

### Standards

NFC operation and interoperability are governed by international standards such as ISO/IEC 14443, which defines proximity card communication, NFC Forum specifications, which ensure compatibility across devices and applications, and JIS X 6319-4, which specifies contactless integrated circuit card requirements, particularly aligned with Japanese transit and payment systems.

### Hardware

NFC systems rely on specialized NFC controller chips integrated into devices such as smartphones, payment terminals, and access control systems. Hardware components include active devices (readers and terminals) that generate magnetic fields and passive or active tags/cards that store data or emulate secure credentials.

### Tools

Development and testing of NFC applications utilize NFC-enabled devices such as smartphones, tablets, and wearables, along with development platforms like Arduino or microcontroller-based NFC shields and modules. Diagnostic and configuration tools include mobile applications such as NFC Tools and NXP TagInfo on Android, which allow reading, writing, and analyzing NFC tags.

### Software

NFC functionality is exposed to developers through software application programming interfaces, enabling integration with operating systems and applications. Examples include the Android NFC framework, which provides APIs for tag discovery, data exchange, card emulation, and secure element access.

### Encryption Keys in NFC

NFC employs encryption keys to secure data communication, ensuring that sensitive information is accessible only to authorized entities during transactions or data exchange.

- Key Types: NFC systems utilize both symmetric and asymmetric encryption keys. Symmetric keys (e.g., AES) enable fast encryption and decryption for bulk data, while asymmetric keys facilitate secure key exchange between devices, ensuring that only the intended recipient can access critical information.

- Key Management: Encryption keys are stored in Secure Elements (SE) or Hardware Security Modules (HSM) to prevent tampering or unauthorized access. Master keys are used to derive working keys for individual transactions, minimizing exposure of high-value secrets.

- Secure APIs: Application Programming Interfaces (APIs) provide controlled access to encrypted NFC data. Only authorized entities, such as issuers or certified developers, can retrieve or decrypt information using the correct key or protocol, ensuring end-to-end security.

Example – Debit/ATM Cards: Banks issue cards with EMV-compliant chips, which implement dynamic tokenization and generate one-time codes for each transaction. NFC terminals access this data only through certified APIs provided by issuers, preventing unauthorized reads. Encryption safeguards sensitive information, including PINs and card details, and restricts data access to approved channels, ensuring secure contactless payments.

## Challenges and Limitations of NFC

NFC is highly secure and convenient for short-range applications, but it is limited by range, speed, storage, environmental factors, and hardware dependency, making it unsuitable for high-volume data transfer or long-distance communication.

| Limitation | Impact |
| --- | --- |
| Short range | Requires very close proximity; not suitable for long-distance applications |
| Low speed | Cannot transfer large files; limited to small data exchanges |
| Limited storage | Restricts use to URLs, small text, tokens |
| Security risks | Vulnerable to relay attacks, skimming, and poorly implemented cryptography |
| Environmental interference | Metal surfaces and electromagnetic noise reduce reliability |
| Hardware dependency | Non-NFC devices cannot participate; adds cost in embedded systems |
| Power dependency | Active devices need sufficient battery |
| Standard fragmentation | Compatibility issues with tags, devices, and regions |

# NFC Based ID cards and Passports

## Introduction

NFC-based ID cards and passports, also known as electronic identity documents and electronic Machine-Readable Travel Documents (eMRTDs), represent a major advancement in modern identity management. These government-issued documents contain a contactless microchip that enables secure, short-range wireless communication using Near Field Communication (NFC) technology. The chip stores personal and biometric information in a standardized digital format, allowing automated and reliable identity verification.

International standards ensure global interoperability: ISO/IEC 14443 governs the contactless communication interface, while ICAO Document 9303 defines the data structure, security mechanisms, and requirements for electronic passports. By combining physical credentials with cryptographically protected digital data, NFC-based ID cards and passports enhance security, reduce identity fraud, and support efficient identity verification for cross-border and domestic applications.

### Why NFC-Based ID Cards & Passports Are Used

- Enhanced Security: The embedded chip stores data in encrypted Data Groups (DGs) with digital signatures in the EF.SOD file. This ensures that any unauthorized modifications are detectable, protecting against forgery and tampering.

- Faster Verification: Contactless communication allows automated systems to read and process identity information quickly, reducing queues at borders and checkpoints.

- Protection Against Identity Theft: Biometric data, such as fingerprints or facial templates, enables secure 1:1 identification. Extended Access Control (EAC) ensures that sensitive data is accessible only to authorized readers.

- Access to Digital Services: Integration with Public Key Infrastructure (PKI) and chip authentication allows for secure use of e-services, such as online banking, e-government portals, and digital voting.

- Compliance with International Standards: These documents follow ICAO Doc 9303 and ISO/IEC 7501, ensuring standardized formats and global interoperability.

- Hygienic and Convenient Processing: Contactless verification reduces physical handling, supporting safe, touchless processing, especially during public health situations. The passive chip typically lasts the document’s validity period, usually 10 years.

## Standards and Terminologies

### Standards

- ISO/IEC 14443: International standard for contactless smart card communication, defining radio frequency, communication protocols, and short-range data transfer for global interoperability.

- ICAO Doc 9303: Global standard for machine-readable travel documents (e-passports), specifying data formats, security mechanisms, and interoperability rules.

- ISO/IEC 19794: Standard for biometric data formats, including facial images, fingerprints, and iris scans, ensuring uniform storage and compatibility across systems.

- ISO/IEC 7501: Standard defining physical characteristics of identity documents, such as size, layout, and durability.

- SHA-256 (Hash Algorithm): Cryptographic function generating unique digital fingerprints of stored data to detect tampering.

- PKI (Public Key Infrastructure): Cryptographic framework for secure verification of identity and digital signatures.

### Terminologies

- Data Groups (DGs): Structured sections of data stored in the chip, each holding specific information: DG1 for personal data, DG2 for facial image, DG3 for fingerprints, and DG4 for iris scans.

- EF.SOD (Elementary File – Security Object Document): File containing digital signatures for all DGs, used to verify data integrity and prevent tampering.

- Passive Authentication (PA): Security method where the reader verifies the authenticity of DGs using the digital signatures in EF.SOD.

- Extended Access Control (EAC): Protocol that restricts access to sensitive biometric data, allowing only authorized readers to access it.

- Logical Data Structure (LDS): Organizational framework that defines how all DGs are stored in the chip for standardized access and global interoperability.

- Chip Authentication (CA): Cryptographic process that verifies the chip itself is genuine, preventing cloning or duplication.

- Hash Algorithms (e.g., SHA-256): Functions generating digital fingerprints of stored data, ensuring tampering can be detected.

## How NFC-Based ID Cards and Passports Work

### 1. Data Storage in the Chip

Each NFC-based ID card or passport contains a secure microchip, which is powered by the reader’s electromagnetic field and does not require a battery. This chip stores information in a Logical Data Structure (LDS), which functions like a digital filing system, organizing all data in standardized sections called Data Groups (DGs).

- DG1: Personal information, including name, date of birth, and ID or passport number.

- DG2: Facial image for biometric verification.

- DG3: Fingerprints (optional), supporting secure identity checks.

- DG4: Iris scans (optional), providing an additional layer of biometric security.

A special file, EF.SOD (Security Object Document), contains digital signatures of all DGs, ensuring that any tampering or unauthorized modification can be detected. Biometric data follows ISO/IEC 19794 standards for universal compatibility.

NFC passport chips (as specified by ICAO Doc 9303) store data in Data Groups (DGs). Here's a complete overview:

ICAO LDS1 (Logical Data Structure) — 16 Data Groups

| DG | Contents | Required? |
| --- | --- | --- |
| DG1 | Machine Readable Zone (MRZ) data | Mandatory |
| DG2 | Facial image (primary biometric) | Mandatory |
| DG3 | Fingerprint(s) | Optional (Extended Access Control) |
| DG4 | Iris image(s) | Optional (Extended Access Control) |
| DG5 | Displayed portrait (visual inspection zone photo) | Optional |
| DG6 | Reserved for future use | — |
| DG7 | Displayed signature or usual mark | Optional |
| DG8 | Data features (document features / security elements) | Optional |
| DG9 | Structure features | Optional |
| DG10 | Substance features | Optional |
| DG11 | Additional personal detail(s) | Optional |
| DG12 | Additional document detail(s) | Optional |
| DG13 | Optional details (country-specific) | Optional |
| DG14 | Security options (EAC/PACE chip auth info) | Optional |
| DG15 | Active Authentication public key info | Optional |
| DG16 | Persons to notify / contact info | Optional |

LDS2 (newer extension, not widely deployed)

LDS2 adds three additional data groups for in-chip travel history:

| DG | Contents |
| --- | --- |
| DG17 | Visa records / entry records |
| DG18 | Border crossing stamps |
| DG19 | Resident permit / additional data |

Supporting Files (not DGs, but stored on the chip)

- EF.COM — Common file listing which DGs are present

- EF.SOD — Document Security Object (digital signatures for integrity verification)

- EF.CardAccess — PACE parameters

- EF.CardSecurity — Chip authentication v2 data

### 2. Data Extraction

When the card or passport is scanned, a reader device sends a short-range radio signal to the chip using ISO/IEC 14443 protocols. The chip responds and transfers data based on authorization level:

- Public DGs (DG1, DG2) can be read directly.

- Sensitive DGs (DG3, DG4) require Extended Access Control (EAC) to ensure only authorized terminals can access them.

This short-range communication ensures privacy and protects against unauthorized reading.

### 3. Authentication and Verification

Multiple layers of security confirm the authenticity of both the chip and its data:

- Passive Authentication (PA): Verifies that the DGs match the digital signatures in EF.SOD.

- Chip Authentication (CA): Confirms that the chip itself is genuine, preventing cloning.

- Extended Access Control (EAC): Ensures sensitive biometric data is only accessible to authorized readers.

- Active Authentication (optional): Some older systems use cryptographic challenges to further confirm chip authenticity.

### 4. Biometric Verification

Once data is extracted and authenticated, the system performs biometric checks:

- 1:1 Matching: Compares the chip’s biometric data (face or fingerprints) with the individual presenting the document.

- 1: N Matching: Compares the biometric data against a database containing multiple records, useful for border security or law enforcement.

### 5. Data Usage

Verified data is used for multiple applications:

- At Borders: Automated Border Control (ABC) systems read the passport within seconds, verify biometrics, and confirm authenticity.

- Digital Services: Integration with PKI allows the chip to digitally sign transactions for secure online services, such as banking, government portals, and e-voting.

- National Systems: ID cards provide secure access to government services, welfare programs, and internal identification systems.

### 6. Security Features

- Encryption: Protects sensitive data stored on the chip.

- Hashing: Algorithms like SHA-256 create digital fingerprints to detect tampering.

- Anti-Collision Protocols: Allow the reader to communicate with a single chip if multiple NFC devices are nearby.

- Short-Range Communication: Limits reading to very close proximity, enhancing security.

## Reading NFC-Based e-MRTDs Using Mobile Phones

Reading an NFC-based passport or ID card with a mobile phone is possible, but only under certain conditions. The device must have NFC hardware and support ISO/IEC 14443 Type A or B protocols. A specialized application is required to read and verify the data stored on the chip.

### Steps for Reading

- Enter MRZ Data: The app asks for information from the Machine-Readable Zone (MRZ), including passport number, date of birth, and expiry date. These details generate cryptographic key Basic Access Control (BAC) to allow secure access.

- Place Passport Near Phone: The chip is powered by the phone’s electromagnetic field, and communication begins using ISO/IEC 14443 protocols.

- Establish Secure Session: All data transfers are encrypted to prevent skimming or unauthorized access.

### Data That Can Be Read

- DG1: Personal information (name, nationality, passport number)

- DG2: Facial image for biometric verification

### Limitations

- DG3 and DG4 (fingerprints and iris scans) are protected by Extended Access Control (EAC) and cannot be accessed by mobile phones.

- Mobile phones cannot modify chip data or bypass cryptographic protections.

### Verification

- The app can check EF.SOD digital signatures and perform Passive Authentication, ensuring the data is genuine and has not been tampered with.

## Limitations of NFC-Based Passport Identification

- Limited Adoption Across Countries: While many countries issue NFC-enabled passports, NFC-based national ID cards are not yet widely implemented globally, limiting cross-border interoperability.

- Hardware Dependency: NFC-based identification requires specialized hardware, such as NFC-enabled readers or NFC-capable smartphones. Users without compatible devices cannot access the system.

- Limited Local Access: In Pakistan, NFC-enabled e-passports are available, but NFC-based ID cards are not. The high cost of e-passports and limited availability of NFC hardware reduce widespread adoption.

- Dependency on Central Authorities: Accessing most personal information (e.g., verification, biometrics) requires interaction with central authorities like NADRA or passport offices. Real-time verification depends on their infrastructure and availability.

- Data Privacy and Security Risks: Sensitive data is protected by cryptographic protocols, but improper implementation or unauthorized hardware could potentially expose information.

- Limited Mobile Access: Mobile devices can read basic personal data and facial images, but access to advanced biometric data (fingerprints, iris scans) is restricted to authorized readers only

# NFC in Banking

Near Field Communication (NFC) in banking refers to the use of short-range, contactless communication technology to enable secure financial transactions, authentication, and identity verification between banking systems, customer devices, and payment terminals. NFC operates at a frequency of 13.56 MHz and allows data exchange between two devices placed within a very short distance, typically less than 4 centimeters, making it suitable for secure banking applications.

Unlike traditional magnetic stripe or chip-and-PIN systems, NFC-based banking systems rely on cryptographic protocols, dynamic transaction data, and secure elements to protect sensitive financial information. Each transaction generates unique, one-time authentication data, ensuring that card details are not reused or exposed during communication. This makes NFC a core technology for modern digital banking, offering a balance of speed, convenience, and high security.

## Applications of NFC in Banking

Near Field Communication (NFC) is widely adopted in banking systems to support secure, fast, and contactless financial services. By enabling short-range encrypted communication between customer devices and banking infrastructure, NFC improves transaction efficiency, security, and user experience. The major applications of NFC in banking are described below.

### Contactless Card Payments

NFC is extensively used in contactless debit and credit card payments. NFC-enabled cards communicate with Point-of-Sale (POS) terminals using EMV standards when tapped within proximity.

Each transaction generates dynamic cryptographic data, ensuring that sensitive card information is not reused or exposed. This significantly reduces risks such as card cloning and skimming while allowing faster transaction processing.

### Mobile Wallet Payments

NFC enables mobile wallet systems where smartphones emulate physical bank cards. Banking applications store payment credentials securely within a Secure Element (SE) or use Host Card Emulation (HCE) combined with cloud-based security.

Before transaction execution, user authentication is performed using biometrics or device credentials, adding an additional security layer beyond traditional card payments.

### Cardless ATM Transactions

Banks use NFC technology to support cardless ATM access. Customers authenticate themselves using a mobile banking application and tap their NFC-enabled phone at the ATM. This method eliminates physical card usage, reducing risks of:

- Card skimming

- Card theft

- Magnetic stripe fraud

Transactions are authorized using encrypted tokens and user verification mechanisms such as PINs or biometrics.

### Secure Customer Authentication

NFC is used as a strong authentication factor in banking systems. Customers can authenticate by tapping their NFC card or NFC-enabled ID on a trusted device during login or account verification. This approach supports multi-factor authentication, combining possession (NFC device) with knowledge or biometrics.

### Digital Onboarding and KYC Verification

Banks utilize NFC to read NFC-enabled national ID cards and passports during customer onboarding. The chip data is accessed securely using standardized cryptographic protocols.

By verifying digital signatures and biometric data, banks can:

- Confirm document authenticity

- Prevent identity fraud

- Accelerate account opening processes

### Virtual Card Issuance

NFC allows banks to issue virtual debit or credit cards instantly. These cards are stored digitally on customer devices and can be used immediately for payments.

Virtual cards reduce dependency on physical card production and provide better control over card usage, limits, and expiration.

### Access Control within Banking Infrastructure

NFC is used for secure access control in banking environments, including branch entry, restricted areas, and employee authentication.

Access rights are managed using encrypted credentials, ensuring that only authorized personnel can access sensitive areas.

### Transaction Authorization and Confirmation

Banks use NFC as an additional verification layer for approving sensitive transactions. Customers may be required to tap an NFC card or device to authorize high-value transfers or configuration changes. This approach enhances security by reducing reliance on SMS-based one-time passwords.

## Working Principle of NFC in Banking

Near Field Communication (NFC) in banking enables secure, short-range, contactless financial transactions and authentication between customer devices (cards, smartphones) and banking infrastructure (ATMs, POS terminals, secure readers). NFC operates at 13.56 MHz, allowing data exchange only when devices are in very close proximity, typically less than 4 cm, which enhances security.

The working principle can be explained in the following steps:

### Electromagnetic Field Generation

- An active device (POS terminal, ATM, or mobile reader) generates an alternating magnetic field using a coil antenna.

- The magnetic field exists in the near-field region and powers passive devices like NFC cards or tags.

### Inductive Coupling and Power Transfer

- When an NFC-enabled card or smartphone enters the magnetic field, inductive coupling occurs.

- Passive devices, such as cards, draw power from the reader’s field, eliminating the need for an internal battery.

- Active devices, like smartphones, generate their own field to communicate with other active devices (peer-to-peer mode).

### Data Exchange Mechanism

NFC communication in banking uses two-way data transfer:

- Reader to Card / Device:
 The POS or ATM modulates the magnetic field to send encrypted commands, such as payment requests or authentication challenges.

- Card / Device to Reader:
 The card or smartphone responds using load modulation, altering its antenna impedance to transmit encrypted transaction data back to the terminal.

### Encryption and Tokenization

- Each transaction generates dynamic cryptographic data to prevent exposure of sensitive information like card numbers or PINs.

- Mobile wallets or NFC-enabled cards store payment credentials securely in a Secure Element (SE) or use Host Card Emulation (HCE) with cloud-based security.

- Tokens and one-time codes ensure that the same card information cannot be intercepted or reused.

### Authentication

- Users authenticate themselves using one or more of the following:

  - PIN or password on the POS/ATM.

  - Biometric verification (fingerprint, face recognition) for mobile payments.

  - Device authentication, where the smartphone or card itself acts as a trusted factor.

- For ATMs, NFC enables cardless access: the user taps a phone or card to the ATM reader, which verifies authorization and initiates secure session.

### Transaction Flow Example – POS Payment

- Customer taps NFC-enabled card or smartphone on the POS terminal.

- Terminal reads encrypted card credentials via NFC.

- Terminal communicates with the bank server or payment network using secure channels.

- The bank validates the transaction, generates a cryptographic approval token, and sends it back to the terminal.

- Terminal confirms transaction to the customer; all sensitive data remains protected.

### Transaction Flow Example – Cardless ATM

- Customer opens the mobile banking app and requests ATM access.

- App generates a secure token for one-time use.

- Customer taps NFC-enabled smartphones at ATM.

- ATM reads the token, validates it with the bank server, and opens the session.

- Customer performs withdrawals or other transactions without a physical card.

### Security Features

- Short-range communication: Reduces risk of eavesdropping or skimming.

- Dynamic encryption: Each transaction uses one-time code to prevent fraud.

- Tokenization: Actual card numbers are never transmitted during NFC transactions.

- Multi-factor authentication: Combines possession (card/phone) with PIN or biometrics.

- Anti-collision protocols: Allow the terminal to communicate with a single card or device when multiple NFC devices are nearby.

### Other Applications in Banking

- Mobile Wallets: Smartphones emulate bank cards to make instant payments.

- Virtual Cards: NFC enables temporary virtual card generation for online or offline use.

- Secure Branch Access: Employees use NFC ID cards to gain access to restricted banking areas.

- Transaction Verification: NFC can act as a second layer of authentication for high-value transactions.

## Limitations of NFC

- Lack of Access to Real Banking Data: Banks do not provide real customer transaction data, EMV keys, or backend details due to privacy and security concerns.

- No Authorization for Full System Implementation: Deploying or testing NFC-based banking solutions in real environments requires approval from banks and regulators, which are not available for research purposes.

- Restricted Access to Card and Secure Element Information: Critical card credentials, secure element data, tokenization mechanisms, and authentication processes cannot be accessed by researchers.

- Simulation-Only Testing: Experiments are limited to dummy data, test cards, or simulated environments; real transactions cannot be conducted.

- Limited Hardware Availability: Certified POS terminals, NFC readers, and secure hardware modules (HSMs) are expensive and not easily available for research purposes.

- Regulatory and Legal Constraints: Financial systems are strictly regulated, preventing deployment of experimental NFC solutions in real banking infrastructures.

- Time and Resource Constraints: Limited time, budget, and access to certified environments restrict full-scale system implementation and testing.

- Cross-Border and Interoperability Issues: Not all countries use NFC-enabled cards or systems, limiting the ability to test international or multi-bank scenarios.

## Conclusion

Near Field Communication (NFC) is a versatile and secure short-range communication technology that has transformed multiple domains, including identity management and banking. Its ability to enable contactless, bidirectional data exchange makes it highly suitable for applications requiring both convenience and strong security.

In NFC-based ID cards and e-passports (eMRTDs), NFC provides a reliable platform for secure storage and transmission of personal and biometric data. These documents enhance identity verification processes at borders, improve access to digital services, and reduce the risks of fraud or tampering through cryptographic protection, tokenization, and standardized protocols. Despite these advantages, limitations such as restricted hardware availability, dependency on central authorities, and controlled access to sensitive biometric data remain significant challenges for researchers and implementers.

In banking applications, NFC enables contactless card payments, mobile wallet transactions, cardless ATM access, and secure authentication, offering faster, safer, and more convenient financial services. The use of dynamic encryption, tokenization, and secure elements ensures that sensitive information remains protected during every transaction. However, research in this field is constrained by lack of access to real banking data, regulatory restrictions, hardware limitations, and dependency on authorization from financial institutions.

Overall, NFC demonstrates substantial potential to modernize identity management and financial systems. Its short-range, secure, and fast communication capabilities make it an essential tool for improving user convenience while maintaining robust security. Future research and implementation efforts should focus on overcoming accessibility, hardware, and regulatory limitations to fully harness NFC technology across global applications.